Name: Maaz Habib

Roll Number: 20PWCSE1952

Section: C

Semester: 3rd

# Lab Report Digital Logic Design Lab

# Department of Computer Systems Engineering

Submitted to: Mr. Faiz Ullah

# Objective of the Lab:

Lab # 8

“Magnitude Comparators”

- Realization of 1-bit comparator using logic gates.

- Realization and implementation of 2-bit comparator using logic gates on breadboard.

- Implementation of 4-bit magnitude comparator on breadboard using IC 7485

- Implementation of 8-bit magnitude comparator using two 7485 ICs.

# Components Required:

- Breadboard.

- IC type 7486 Quadruple 2-input XOR gates.

- IC type 7408 Quadruple 2-input AND gates.

- IC type 7400 Quadruple 2-input NAND gates.

- IC type 7410 triple 3-input NAND gates.

- IC type 74L85 4-bit magnitude comparator.

- Switches for inputs.

- LED displays for outputs.

# Theory:

Magnitude Comparator:

Magnitude Comparator are digital circuits which have two ports which accept and have three single bit outputs. It is used to comparing individual bits, multi-bit comparators can be constructed to compare whole BCD words to produce an output if one word is larger, equal to or less than the other.

### Block Diagram:

#### Figure 1: Block diagram of n-bit Magnitude Comparator

3-bit comparator:

It is a comparator used to compare two 1-bit binary numbers. It has two binary inputs A, B and three binary outputs: greater than, equal and less than relations.

### Block Diagram:

### Truth Table:

Expressions:

#### Figure 2

| INPUTS |  | OUTPUTS |  |  |
| --- | --- | --- | --- | --- |
| A | B | A=B | A<B | A>B |
| 0 | 0 | 1` | 0 | 0 |
| 0 | 1 | 0 | 1 | 0 |
| 1 | 0 | 0 | 0 | 1 |
| 1 | 1 | 1 | 0 | 0 |

Truth table of 1-bit comparator

The Boolean functions describing the 1-bit magnitude comparator according to the truth table are:

- (𝑨 = 𝑩) = 𝑨̅. 𝑩̅ + 𝑨. 𝑩 = ̅(̅𝑨̅̅̅⊕̅̅̅𝑩̅̅̅)

- (𝑨 < 𝑩) = 𝑨̅. 𝑩

- (𝑨 > 𝑩) = 𝑨. 𝑩̅

# 2-bit Magnitude Comparator:

It is a comparator used to compare two 2-bit numbers. It has 4 binary inputs (number A: A1A0, number B: B1B0) and 3 binary outputs: greater than, equal and less than relations.

### Block diagram:

Figure 4 shows the block diagram of 2-bit magnitude comparator.

### Truth table:

Expressions:

#### Figure 4

| INPUTS |  |  |  | OUTPUTS |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| A1 | A0 | B1 | B0 | A>B | A=B | A<B |
| 0 | 0 | 0 | 0 | 0 | 1 | 0 |
| 0 | 0 | 0 | 1 | 0 | 0 | 1 |
| 0 | 0 | 1 | 0 | 0 | 0 | 1 |
| 0 | 0 | 1 | 1 | 0 | 0 | 1 |
| 0 | 1 | 0 | 0 | 1 | 0 | 0 |
| 0 | 1 | 0 | 1 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 | 0 | 0 | 1 |
| 0 | 1 | 1 | 1 | 0 | 0 | 1 |
| 1 | 0 | 0 | 0 | 1 | 0 | 0 |
| 1 | 0 | 0 | 1 | 1 | 0 | 0 |
| 1 | 0 | 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 1 | 0 | 0 | 1 |
| 1 | 1 | 0 | 0 | 1 | 0 | 0 |
| 1 | 1 | 0 | 1 | 1 | 0 | 0 |
| 1 | 1 | 1 | 0 | 1 | 0 | 0 |
| 1 | 1 | 1 | 1 | 0 | 1 | 0 |

Truth table

Using key-map, the simplified Boolean function for the outputs A>B, A=B and A<B is shown below:

### For A=B:

#### A=B:

= 𝑨̅𝟏. 𝑨̅𝟎. 𝑩̅𝟏. 𝑩̅𝟎  + 𝑨̅𝟏. 𝑨𝟎. 𝑩̅𝟏. 𝑩𝟎  + 𝑨𝟏. 𝑨̅𝟎. 𝑩𝟏. 𝑩̅𝟎  + 𝑨𝟏. 𝑨𝟎. 𝑩𝟏. 𝑩𝟎

= (𝑨̅𝟏𝑩̅𝟏  + 𝑨𝟏𝑩𝟏). (𝑨̅𝟎𝑩̅𝟎  + 𝑨𝟎𝑩𝟎)

= (𝑨𝟏 ⊕ 𝑩𝟏)′. (𝑨𝟎 ⊕ 𝑩𝟎)′

= 𝑿𝟏. 𝑿𝟎

### For A>B:

#### A>B:

= 𝑨𝟏𝑩′ + 𝑨′ 𝑨𝟎𝑩𝟏′𝑩𝟎′ + 𝑨𝟏𝑨𝟎𝑩𝟏𝑩′

𝟏	𝟏	𝟎

= 𝑨𝟏𝑩′ + 𝑨𝟎𝑩′ (𝑨′ 𝑩𝟏′ + 𝑨𝟏𝑩𝟏)

𝟏	𝟎	𝟏

= 𝑨𝟏𝑩′ + 𝑨𝟎𝑩′ (𝑨𝟏⨁𝑩𝟏)′

𝟏	𝟎

= 𝑨𝟏𝑩′ + 𝑿𝟏. 𝑨𝟎𝑩′

𝟏	𝟎

### For A<B:

#### A<B:

𝟏

= 𝑨𝟏′𝑩𝟏 + 𝑨′ 𝑨𝟎′𝑩𝟏′𝑩𝟎 + 𝑨𝟏𝑨𝟎′𝑩𝟏𝑩𝟎

𝟏

= 𝑨𝟏′𝑩𝟏 + 𝑨𝟎′𝑩𝟎(𝑨′ 𝑩𝟏′ + 𝑨𝟏𝑩𝟏)

= 𝑨𝟏′𝑩𝟏 + 𝑨𝟎′𝑩𝟎(𝑨𝟏⨁𝑩𝟏)′

= 𝑨𝟏′𝑩𝟏 + 𝑿𝟏. 𝑨𝟎′𝑩𝟏

# 4-bit Magnitude Comparator:

A comparator used to compare two 4-bit words. The two 4-bit numbers are word A: A3A2A1A0, and word B: B3 B2B1B0) so, the circuit has 8 inputs and 3 binary outputs: A>B, A=B and A<B.

Block Diagram:

Pin Description:

#### Figure 6(a)

Figure 6(b)

### Working of 4-bit comparator:

Equality:

- Word A equal word B if: A3=B3, A2=B2, A1=B1, A0=B0.

### Inequality:

- If A3 = 1 and B3 = 0, then A is greater than B (A>B). Or

- If A3 and B3 are equal, and if A2 = 1 and B2 = 0, then A > B. Or

- If A3 and B3 are equal & A2 and B2 are equal, and if A1 = 1, and B1 = 0, then A>B. Or

- If A3 and B3 are equal, A2 and B2 are equal and A1 and B1 are equal, and if A0 = 1 and B0 = 0, then A > B.

- If A3 = 0 and B3 = 1, then A is less than B (A<B). Or

- If A3 and B3 are equal, and if A2 = 0 and B2 = 1, then A < B. Or

- If A3 and B3 are equal & A2 and B2 are equal, and if A1 = 0, and B1 = 1, then A<B. Or

- If A3 and B3 are equal, A2 and B2 are equal and A1 and B1 are equal, and if A0 = 0 and B0 = 1, then A < B.

# Lab Tasks:

Part A:

## Procedure:

- Check all the components for their working.

- Insert the appropriate ICs into the IC base.

- Make connections as shown in the circuit diagram in figure 5.

- Verify the Truth Table and observe the outputs.

- Repeat the same steps but for the circuit diagram in figure 6 and apply inputs in the table. Record the outputs for the given values of A and B.

## Table:

| INPUT |  | OUTPUT |
| --- | --- | --- |
| A | B |  |
| 10 | 01 | A>B |
| 11 | 11 | A=B |
| 00 | 01 | A<B |
| 01 | 01 | A=B |

For 4-bit Magnitude Comparator:

## Table:

| INPUT |  | OUTPUT |
| --- | --- | --- |
| A | B |  |
| 1001 | 0110 | A>B |
| 1100 | 1110 | A<B |
| 0011 | 0101 | A<B |
| 0101 | 0101 | A=B |

Conclusion:

So, Magnitude Comparator is studied.

# Part b:

- Design an 8-bit comparator using two chips of IC 7485. The connections are given below.

- Verify the given truth table.

## Pin description:

Logic diagram for 8-bit Magnitude Comparator:

## Truth Table:

| INPUTS |  | OUTPUTS |  |  |
| --- | --- | --- | --- | --- |
| A | B | A>B | A=B | A<B |
| 0000 0000 | 0000 0000 | 0 | 1 | 0 |
| 0001 0001 | 0000 0000 | 1 | 0 | 0 |
| 0000 0000 | 0001 0001 | 0 | 0 | 1 |

Verification of Truth Table: